Evaluate a complex function expressed in complex spherical harmonics at a set of

points.

Usage

-----

value = MakeGridPointC (cilm, lat, lon, [lmax, norm, csphase, dealloc])

Returns

-------

value : complex, ndarray

Value of the function at (lat, lon).

Parameters

----------

cilm : complex, dimension (2, lmaxin+1, lmaxin+1)

The complex spherical harmonic coefficients of the function. The

coefficients C0lm and C1lm refer to the positive and negative angular

orders, respectively, with Clm=cilm[0,1,m] and Cl-m=cilm[1,l,m].

lat : float, array_like

The latitude of the point in degrees.

lon : float, array_like

The longitude of the point in degrees.

lmax : integer, array_like, optional, default = lmaxin

The maximum spherical harmonic degree used in evaluating the function.

norm : integer, array_like, optional, default = 1

1 (default) = Geodesy 4-pi normalized harmonics; 2 = Schmidt semi-normalized

harmonics; 3 = unnormalized harmonics; 4 = orthonormal harmonics.

csphase : integer, array_like, optional, default = 1

1 (default) = do not apply the Condon-Shortley phase factor to the

associated Legendre functions; -1 = append the Condon-Shortley phase factor

of (-1)^m to the associated Legendre functions.

dealloc : integer, array_like, optional, default = 0

0 (default) = Save variables used in the external Legendre function calls.

(1) Deallocate this memory at the end of the funcion call.

Description

-----------

MakeGridPointC will expand a complex function expressed in complex spherical

harmonics at a set of points. The input latitudes and longitudes are in degrees,

and the maximum degree used in evaluating the function is the smaller of lmaxin

and lmax. The employed spherical harmonic normalization and Condon-Shortley

phase convention can be set by the optional arguments norm and csphase; if not

set, the default is to use geodesy 4-pi normalized harmonics that exclude the

Condon-Shortley phase of (-1)^m.